AUTOFOCUS

M and X SERIES LASER SYSTEMS

Installation and Operation

Instructions

06/01/98

These instructions assume that you already know how to operate the laser system.  If you do not know how the laser system works,  please read to the Operations Manual completely before continuing.  A complete understanding of different focusing techniques as well as the operational aspects of the machine is of vital importance.

To Turn On AUTOFOCUS:

- Power up the machine.

- When “READY” appears,  press the “ESCAPE” key.

- Using the down arrow key,  scroll the cursor down to the “PREFERENCES” line and press “SELECT”.

- Press “SELECT” again when the cursor is on the “CONFIGURATION” line.

- Using the down arrow key,  keep scrolling down until reaching the line “AUTO FOCUS   OFF”.

- While the cursor is on this line,  press the “SELECT” key to toggle it to “ON”.

- Press the “ESCAPE” button several times to bring you back to the main menu.

To Use AUTOFOCUS:

- The AUTOFOCUS sensor sends out an invisible beam across the table between the 1 inch and the 3 inch mark in the Y axis ruler for the M series and the 2 and 4 inch mark for the X series laser systems.  You will notice that it is indented slightly.

- Place your object between these two points vertically and anywhere on the table horizontally.  Refer to the diagram below.

- Make sure that the laser system is in the main menu screen or the “File Display” menu. AUTOFOCUS will not work in the other menus.

- Press the “Z” button once on the control panel of the laser system.  You will observe the Z-axis Table (Engraving Table)  move down and then up again,  adjusting the table to the proper focusing height.  If the “Current Lens” choice in the menu system is not the same as the lens you have installed,    correct it by placing the cursor on the second line of the display,  and pressing the “Select” button to toggle you through your choices. Once the desired lens is displayed,  escape out of the menu and AUTOFOCUS again.  This will bring the table up to the correct height.  The display will now show that the “Z Position” is “0.00” for that Focus Lens.

- If you desire to adjust the “Z Position” so that you are slightly out of focus,  without exiting out of the menu,  simply bring the cursor over to the “0.00” part of the display and adjust the table to the desired offset.  The display will now show you how much you are out of focus.  If you were to escape out of the display and perform the AUTOFOCUS procedure again,  the “Z Position” will again show “0.00” to remind you that you are precisely in focus.

- If you have purchased additional lenses besides the 2.0 inch lens upon original delivery of the laser system,  those other lens heights have been set at the factory.  If you purchase additional lenses after you have received your machine and need to set the ‘Focal Height” of that lens,  or you need to adjust your AUTOFOCUS, please follow the next procedure.

To Adjust AUTOFOCUS:

From time to time,  you may need to re-adjust your AUTOFOCUS.

- Make sure that nothing is underneath the Engraving Table.

- Re-home the Z-axis by pressing the “Z” button,  scrolling down to the bottom line that states “RE-HOME Z AXIS”, and pressing the “Select” button.

- The Z-axis will move all the way down to the bottom and the Focus Carriage will move to the upper right hand corner of the laser system.

- Place a very flat object on the Engraving Table within the AUTOFOCUS Zone.

- AUTOFOCUS on this object by pressing the “Z” button once.

- While you are still in the “Z Position” display,  move the Focus Carriage using the motion control keys so that it is above your object that you placed on the table.

- Place your Focus Tool on top of your object and compare the Focus Tool reading with the actual height that the AUTOFOCUS set the Engraving Table to.  If it is not the same,  bring the cursor up to the first line of the “Z Position” display and press the “Select” button.  Now focus on your object by using your Focus Tool and adjusting the “Z Position” until your are manually focused on top of your object.

- Press the “Escape” button,  move the cursor to the “Set Focal Length” line and press “Select”.  It will ask you if you are sure so respond accordingly.  The AUTOFOCUS will now have been re-adjusted to be exactly at the same height as your Focus Tool.  The purpose of “Setting the Focus Length” is to calibrate AUTOFOCUS with your Focus Tool so that whether you use AUTOFOCUS or use the Focus Tool,  you will always bring put the Engraving Table at the correct focusing height.

The only time that you should need to re-adjust the AUTOFOCUS is when you have received a new or different length Focus Lens, or have recently Flash upgraded the operating system of the laser.

Maintenance

You must keep the Reflector and the AUTOFOCUS Sensor clean.  Engraving smoke or debris that will build up on these devices will render it inoperative.  If these items are dirty,  what will happen is that the Engraving Table will drive itself all the way down to the bottom of its travel if you try to use the AUTOFOCUS option.  Clean the Sensor and the Reflector with a clean,  soft cloth or tissue lightly dampened with water.  A mild soap solution is OK only if you just dampen your cloth lightly and wipe across the surface of the Sensor and Reflector.  NEVER SPRAY OR POUR ANY FLUIDS ONTO THE SENSOR.  The sensor is an electronic device and can be destroyed if you are careless.

-------------------------------------------------------------------------------------------------------------------

If you have any concerns or questions about how to operate or use this option,  please contact your Sales Representative or the Sales/Applications Department at Universal Laser Systems Corporate Headquarters.

Universal Laser Systems

16008 North 81st Street

Scottsdale,  Arizona  85260  USA

Phone: 602-483-1214

Fax: 602-483-5620

email: sales@ulsinc.com

-------------------------------------------------------------------------------------------------------------------

PLEASE DO NOT CONTACT THE SERVICE DEPARTMENT REGARDING APPLICATIONS OR SYSTEM USAGE QUESTIONS.  PLEASE CONTACT YOUR SALES REPRESENTATIVE OR THE SALES/APPLICATIONS DEPARTMENT AT UNIVERSAL LASER SYSTEMS.

If your AUTOFOCUS is broken or does not function,  please contact Universal Laser Systems Service Department at:

Universal Laser Systems Service Department

7707 East Acoma Drive #108

Scottsdale,  Arizona  85260   USA

Phone: 602-609-0297

Fax: 602-609-1203

- email: service@ulsinc.com