Dual Head

INSTALLATION and OPERATION

The following instructions assume that you fully understand how to operate the laser system.  Do not attempt to use the Dual Head option without complete knowledge about how the laser system works. The purpose of this option is to engrave two of the same images at the same time to enhance throughput (not twice as fast). Although you are engraving with a split beam, remember that your power is cut in half for each head so you will need to cut your speed in half to produce the same depth as a single head.

- Turn on the laser system.

- Select the “Dual Head” option in the control panel.

- It will instruct you to turn off the laser system and wait at least 15 seconds.

- Before turning the system back on,  install the Dual Head parts. Refer to the diagram below.

- If your system has air assist, connect the Dual Head’s coiled air hose to the back of the X-axis arm by loosening the two screws, removing the block, and replacing it with the block that is attached to the coiled air hose.

- Turn the laser system back on.

- In the driver, select the “Dual Head” option. You will notice that the field size is now cut in half.

- Adjust the page size in your graphics program to match the page size in the driver.

- The field and the laser’s power is cut in half. Adjust your settings accordingly to get the desired effect.