ULS XL.12000, experience from a frustrated owner. (Sorry, but there is more later.)

The laser is operated at temperature 25 - 27 Celsius.

The air is dry (Freezer-dryer).

- ROTARY FIXTURE & AIR ASSIST. – This is an engineering issue and has been passed to that department.

a.	It is nowhere in the manual told that this do not work together.

b.	If it is not meant to work together, then why is the software not asking to remove the air-cone? The software can see that a rotary fixture and an air-cone are mounted.

c.	When the air-cone is mounted, and auto focus is selected, the cone gets stuck on top of the rotary motor, and it is almost impossible to get it free.

- When the camera has been selected it often causes problems in the job hereafter.

d.	An error: no communication; the head is impossible to move and you have to hit the breakers, and reboot. – Is the carriage allowed to return home or is it pushed to the home position after the system is auto-focused?

It is not pushed home. When the error occur, you are not able to move the carriage after a reboot. You have to move the carriage when the laser is shut off, and the reboot again

e.	The system is not homing, and starting the job from the current position, and if you have the material placed in the left side there will be very noisy movements, when the head hits the left side of the machine. – Is this as soon as the engraver is turned on, or is it after a job has been run on the engraver? After another job

- Cutting table. -

f.	The cutting table is almost impossible to remove. I have to unscrew the near left mounting-set. The flanges must have bent in shipping. Use a pliers or other tool to gently bend the mounting flanges back into shape.

g.	The left ruler is pointing upwards, so 2mm acrylic will glide in under the ruler. – The honeycomb tends to flex a little if it is not exactly cut to shape. Remove the ruler and re-adjust or re-fit the honeycomb to the table. The comb can be cut with a pair of scissors or formed with gloved fingers. This would be the same procedure if you had to replace the honeycomb.

The left ruler is still pointing upwards. It is not in a 90% angle with the table.

h.	The honey-comb material is not fastened to the table, so the material is not even. Some areas are out of focus because the honey-comb material is “floating”. – This may be fixed once the comb is adjusted as described above.

- Erratic.

i.	The system freezes in the middle of the job. There is a picture on the screen, but no action. The only possibility is to hold the on button to shut down. – Turn the Vector Enhancements to Enhance only on the driver.

j.	The system stops in the middle of the job with an error concerning no response from USB device. As I am informed, there is no USB on the system – This error means that the system has lost communication with the air assist.  There is an air assist update in the works.

k.	Sometime when no computer is connected to the LAN the system shows a graphic (can’t tell what it is suppose to show) on the screen and will not boot.- This needs more clarification. The system was designed to work independently from a PC after the files were downloaded and stored on the engraver. This is not permanent, I will take a picture of the grafichh next time.

l.	When booting, and after, there is sometimes no picture on the screen. – There is a 20 second delay from when power is applied to when the power button should be pressed on the engraver.

- When looking in the log, there are many entries with Hardware error. This has to do with 4a. 4a ????

- When changing the z-axis there sometime is a noise like the motor having trouble moving the table. – The Z axis position needs to be re-homed. How often?

- Sometime, when auto focusing; pressing the camera button twice, the z-axis moves and the air-cone gets stuck in the material, and you have to press the emergency button.. The material is a large piece of acrylic; surface is bigger than the carriage. – The Z axis position needs to be re-homed and may need to be reset. How often ?

m.	Where is the focus censor, and has it to be cleaned.- Just to the left and slightly behind the opening for the cone. Has it to be cleaned?

- Manual. – The manual is being updated.

n.	I can not find anything about

i.	Air assist, mounting and usage. PSI requirements etc.

ii.	HPDFO usage

iii.	How to align the carriage, when it is misaligned from getting the air-cone getting stuck in the acrylic. (Moving from left to right, the line is not parallel with the horizontal ruler.)

iv.	Rotary fixture. Mounting and usage.

- Poor response on calls regarding problems.

o.	Are there online forms to be filled out, to document the users calls? As long as there is warranty, there should be a way to communicate the problems to ULS. I have not got any official answers on my calls, but one, where I was called from ULS USA, and was promised a written procedure the next day. Still waiting.