AIR ASSIST MESSAGES:

Please install the cone before using Air Assist.

Insufficient flow from air/gas assist device.

No 48V power detected from air assist device.

Air/gas assist not conneced.

Error reading data from the Compressor.  Engraving aborted.

Compressor USB Error:  Not Detected. [These last 3 are probably all USB errors.]

AUTO-FOCUS MESSAGES:

Auto-Focus Fails! [Didn’t see the LED change during autofocus].

BLACK BOX MESSAGES:

No Air [Means the Air Asisst electronics not detected.  Might be a USB error.]

Comm Api

Comm Bytes

Comm Read

Comm Magic

Comm Write

[All of these are USB communication I/O errors.  The words “Api”, “Bytes”, etc., are codes that indicate to us which function(s) in the application logged the error.  What could cause these?  Might be a bad cable, bad connector, bad VIA board.  Getting one of these once in a while is not necessarily fatal to the machine’s proper operation.]

No Camera	[Camera electronics not detected.]

No Processor	[No communication at all to the engraver electronics.]

No Keypad	[Keypad USB error.]

CONNECTION MENU MESSAGES:

Duplicate Address [The guy has entered an IP address that already exists on his network.]

DOOR MESSAGES:

The Top Door is open.

FIRMWARE/SOFTWARE INSTALLATION MESSAGES:

We've detected a mis-matched firmware version.  You may need to shut down the Engraver altogether (i.e., push Red Knob on front panel) to complete the firmware upgrade.

Incorrect USB Driver Version  [This happens when the app first starts up, and it occurs if we’ve installed the wrong version of ULSPRINT.SYS on the engraver.  Usually not the customer’s fault.]

Please initialize the Printer Model.  The printer model is unknown. [This happens only when we’re upgrading an original XL electronics board to a current board.  I don’t know if we’ve upgraded all the installations yet.]

LENS MESSAGES:

The Engraver cannot detect a proper lens installed.  Please check the optics. [This happens when the lens magnets are incorrect or aren’t attached properly.]

MOTION FAULT MESSAGES:

Collision Detected.  Press OK to re-home.

End of Travel Detected

Please move the laser carriage by hand away from the side of the machine. [I think this one happens only when you’re exiting the Focus/Camera screen.  We’ve had a motion fault while the motors were de-energized, but we can’t energize the motors again until the user moves the carriage out of the faulting location.]

Head Crash [This happens if the laser carriage is pushed off-kilter, like when it runs into something vertically.]

Head Crash Detected.  Press OK to re-home. [Same as above.]

PRODUCTION STARTUP ERRORS:

Please home the Z-axis before calibrating the Z-height.

Please remove the Cutting Table fixture before setting the Z-Axis.

Please Initialize the Auto-Focus Z-Position

Please Initialize Laser Watts

Please Initialize R-Focus Position.

Please Initialize 2.0" Lens Position.

Please Initialize 3.0" Lens Position.

Please Initialize HPDFO Lens Position.

Please initialize the maximum Z-platform height.

The Rotary Y-axis offset value is zero.  Please initialize the Rotary Offset value.

[If we get one of these, it means the board wasn’t properly initialized during production.  It might happen if we send a new hard drive (or Cypress board) to an existing customer and replace it in the field.]

ROTARY-TYPE MESSAGES:

The Engraver detects a Rotary fixture installed, but the Print Job was not generated for a Rotary fixture.  Please re-print the file, or else replace the Rotary fixture with some other platform.

This Print Job was generated for a Rotary fixture, but the Engraver does not detect a rotary fixture attached.  Please re-print the file, or else attach the Rotary fixture.

The Engraving Table is too high for the Rotary Fixture.

Unknown Fixture [We’re detecting a bogus fixture installed; not a Rotary, not a Standard table.]

The Print Job contains an unrecognized table fixture code. [This is an error in the print job, not the engraver; probably caused by a software bug in the printer driver.]

A fixture or table appears to be incorrectly attached to the Engraver's engraving platform.

Z-HOMING MESSAGES:

Processor Failure [Error that might occur during Z-homing; not sure what would cause this.]