# Error Messages XL Series Engraver

| ERROR | CAUSE | SOLUTION |
| --- | --- | --- |
| Air/Gas Assist not connected. |  |  |
| Batteries. | Fire detection batteries in need of replacing. | Remove rear electronics panel. Replace the 3-9 volt batteries used for fire detection. |
| Collision Detected. | Collision Detector sensed material above focal height. | - Inspect mark area for obstruction. - Press OK to re-home. |
| Compressor USB not detected. | - Communication between Air Assist Board and CPU interrupted. - USB Air Assist Board defective. | - Recycle power to the system. - Contact ULS Service. |
| Duplicate Address. | IP address was entered that already exists on his network. | Contact your System Administrator for alternative IP Address. |
| End of Travel Detected. | Caused by motion fault while the motors were de-energized. | Move the laser carriage by hand away from the side of the machine. |
| Engraver cannot detect a proper lens installed. | Lens magnets are incorrect or aren’t attached properly. | - Inspect lens for magnet, if attached clean. - Operation of the system is possible without the use of Auto Z or Auto Focus and by focusing with the focus tool. - Contact ULS Service. |
| Front Door Open. | - Interlock condition. - Sensor failed. | - Clean and align sensors. - Jumper sensors on wiring harness side to determine which sensor has failed. |
| Insufficient Flow Air / Gas assist device. | - Compressor valve is closed. - Filters are plugged. - Compressor pressure below 50 PSI. - Defective Air Assist flow valve. | - Verify all valves are open. - Check desiccant filter and coalescing filter to see if plugged. - Adjust pressure 50-60 PSI. - Contact ULS Service. |
| No Camera. | Camera electronics not detected. | - Option not installed. - Contact ULS Service. |
| No Keypad. | Keypad USB error. | Contact ULS Support. |
| No Processor. | No communication at all to the engraver electronics. | Contact ULS Service. |
| No 48V power detected from air assist device. |  |  |
| Please install the cone before using Air Assist. | Air Assist turned on in printer driver with no Air Assist Cone or Back Sweep installed. | Install Air Assist Cone or Back Sweep option or turn air assist off in printer driver. |
| ERROR | CAUSE | SOLUTION |
| Top Door Open. | - Clean and align sensors. - Sensor failed. | Jumper sensors on wiring harness side to determine which sensor has failed. |